Project Title: SafeSip  	                                 Date: 10/10/2025

Overview (Provide a brief summary of your project idea and the background.)

Background:

In regions like Kisii, Kenya, families collect water from rivers, springs, and rain tanks. After rainfall, water often becomes murky and unsafe to drink, while storage in jerrycans leads to re-contamination. Laboratory testing kits are expensive and difficult to use. SafeSip addresses this limitation by guiding users through what to do next and confirming that their water stays safe even after treatment.

What is the project about?

SafeSip is a portable Arduino-based water quality device that measures pH, then advises whether the user should boil or chlorinate the input of water. It also verifies the effectiveness of chlorination by reading the pink color of a DPD chlorine test strip using a color sensor. Additionally, it includes a small sampling wand that allows users to test water stored in jerrycans later to ensure it remains safe to drink.

Project Objectives (Define the goals of your project. What do you want to achieve? What problem does it solve, or what creative experience does it provide?)

This project aims to create a low-cost, offline water advisor that transforms raw sensor readings into simple, understandable guidance. The device helps users identify which treatment method is effective depending on the day’s water conditions and environmental factors. It ensures chlorination worked correctly by verifying free-chlorine levels and prevents re-contamination during storage by letting users recheck water using the sampling wand. By combining technical sensors with clear visual indicators and local-language instruction cards, SafeSip promotes practical, long-term water safety habits in rural areas like Kisii.

Target Audience

Who will benefit from or interact with this project? (General consumers, students, hobbyists, etc.)

SafeSip is designed for everyday families in rural communities, especially in regions like Kisii where access to clean water is inconsistent. It also benefits community health workers, students, emergency responders, and campers who need a quick, offline method to test and verify water safety without laboratory tools or internet access.

Project Components

Key Electronics

- Arduino UNO R4 Minima — Central processor.

- pH Measurement (replaced for faster shipping): Atlas Scientific EZO-pH circuit + lab-grade pH probe (Amazon) — Stable, accurate, I²C/UART interface.

- AS7341 Spectral/Color Sensor (Adafruit) — Reads pink intensity on free-chlorine test strips to verify residual.

- LED indicators (Green/Red) —  Green means safe, Red means not safe. LEDs will also have flashing patterns to signal treatment method + optional buzzer.

- Push Button (16 mm) — Start

- Resistors (e.g., 220–330 Ω for LEDs; 4.7 kΩ for DS18B20 pull-up).

- Breadboard & Jumper Wires — No-solder prototype.

- USB cable + 5 V power bank — Portable power.

Enclosure & Hardware

- 3D-Printed Enclosure (PETG/PLA). Left: electronics; Right: splash-isolated sample cup.

- Removable Plastic Cup (sample well).

- 3D-Printed Light-Baffled Slot for strip reading over the AS7341.

- Sampling Wand: Slim 3D-printed dipper head + short food-grade silicone tube and a one-way check valve to capture a small spout-level sample from a 20 L jerrycan without removing the cap fully.

Calibration Materials

- Free-Chlorine Test Strips (Hach 2745050) — DPD chemistry built in; used for Verify step (no powder reagents needed).

- pH Buffer Solutions (pH 4.00, 7.00, 10.00) — For pH calibration (Atlas probe).

- Laminated Dosing Card (English/Ekegusii/Swahili) — Baseline drops per 20 L jerrycan by bleach strength (e.g., 3.5% vs 5%), contact time, and SODIS rules.

Functional Description

Explain how the project will function. Describe the interaction between components, the user experience, and workflow.

Input Sensors:

- pH: Determines chlorine effectiveness (lower pH → stronger HOCl; high pH weakens chlorination).

- Color (AS7341): Reads free-chlorine strip color to confirm residual in the safe band.

Output Actions:

- LED Indicators: Different patterns of Green LED & Red LED flashing will indicate whether to use chlorination or boiling (key will be provided with prototype to interpret flashing)

- Chlorination Verification: After treatment, the device confirms proper chlorine levels by scanning the test strip; if insufficient chlorine is detected, the red LED blinks to prompt additional dosing. (each red LED blink will indicate # of additional 0.5 ml chlorine drops, chlorine mg per liter must be at least 0.2 mg but should not exceed 1.0 mg – whole 20 liter jerrycan would require 4-8 drops).

- Re-Verification: Using the sampling wand, the user can recheck stored water; if it remains safe, the green LED flashes, but if chlorine has faded, the red LED warns the user to re-dose.

Block Diagram Provide a simple visual representation of your project’s system. Include key components like inputs (sensors), outputs (actuators), and connections.

Challenges and Risks (Identify potential technical or creative challenges that might arise.)

Coding difficulty: Combining multiple analog sensors through an ADS1115 interface and coding the logic to correctly compare safe thresholds.

Hardware challenge: Preventing leaks or water damage to electronic parts and ensuring accurate calibration of sensors and stable readings.

Time constraint: 3D printing and sensor calibration can take several days to perfect. Testing may require multiple iterations before achieving consistent results.

References/Inspirations

List ALL resources, tutorials, or previous projects that have inspired or guided your proposal.

DFRobot Gravity pH Sensor Kit V2 (SEN0161-V2) – Wiki

DS18B20 Temperature Sensor with Arduino – Guide (Random Nerd Tutorials)

Arduino UNO R4 Minima – Official Docs / Pinout / Getting Started

EPA: National Primary Drinking Water Regulations (standards overview)

CHECKLIST

Complete all of the above question

Complete the component sheet. Resource Request Sheet (make a copy and delete the data included)

Before you submit this doc, do not forget to attach the Resource Request Excel file with this.